# Termite Treatment - Complete Guide

# Types of Termites in Pakistan

Subterranean Termites: Most common type, live underground and build mud tubes

Drywood Termites: Live inside wood, no soil contact needed

Dampwood Termites: Found in moist, decaying wood

Formosan Termites: Highly destructive, large colonies

# Treatment Methods

Eastern Services uses advanced pre-construction and post-construction anti-termite treatments. Pre-construction treatment involves soil treatment with Termidor SC or Premise 200 SC before foundation is laid. Post-construction treatment includes drilling and injecting termiticide around the foundation, treating all wooden structures, and creating a chemical barrier. Treatment is effective for 5-10 years depending on soil conditions and building structure.

# Signs of Termite Infestation

Mud tubes on walls or foundation

Hollow-sounding wood when tapped

Discarded wings near windows or doors

Frass (termite droppings) that look like sawdust

Bubbling or uneven paint

Tight-fitting doors or windows

# Treatment Duration and Process

Complete termite treatment takes 1-3 days depending on property size. Process includes: inspection (1-2 hours), drilling holes around foundation (3-4 hours), chemical injection (2-3 hours), treating wooden structures (2-4 hours), and sealing drill holes. Property can be occupied during treatment with minimal disruption.